TENDER SPECIFICATIONS

This unit varies the speed of a DC motor for control applications.

Metallic box.

Front panel including:

Excitation terminals:

Connection with the excitation of the DC Motor.

Red Terminal / Positive Polarity. Black Terminal / Negative Polarity.

Rotor terminals:

Connection with the rotor of the DC Motor.

Red Terminal / Positive Polarity. Black Terminal / Negative Polarity.

Tachodynamo terminals:

Connection with an external DC Tachogenerator.

Red Terminal / Positive Polarity. Black Terminal / Negative Polarity.

Torque control: Control of the mechanical torque of the motor.

Speed control: Control of the speed of the motor.

Start/Stop switch to run or stop the motor. On/Off switch to turn the unit on.

Feedback switch: switch to control the feedback source (tachodynamo or internal).

LED indicators of the status of the unit.

Maximum power: 550W.  Maximum torque: 2.6 Nm.  Speed range: 130-2000 r.p.m.

Besides the motor speed controller, all the indicators, connecting cables and additional components are included.

Electrical supply required: 220 V a.c. – 110 V a.c., single-phase + ground.

Dimensions: 300 x 190 x 120 mm. approx.  Weight: 10 kg. approx.

EXERCISES AND PRACTICAL POSSIBILITIES

1. Motor parameters adjustment

2. Working at a certain frequency.

3. Starting carrying out by defined ramp.

4. Braking possibility until a certain speed and recovery.